- SIGNAL GENERATION AND BASIC SIGNAL OPERATIONS

Experiment 3.1

Addition and Multiplication of Two Sequences

Program:

clc;

clear;

close all;

% Sample index

n = 0:10;

% Sequence 1

x1 = [1 2 3 4 5 6 7 8 9 10 11];

% Sequence 2

x2 = [11 10 9 8 7 6 5 4 3 2 1];

% Addition of sequences

y_add = x1 + x2;

% Multiplication of sequences

y_mul = x1 .* x2;

% Plotting

figure;

subplot(2,2,1);

stem(n,x1,'filled');

grid on;

title('Sequence x_1(n)');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,2);

stem(n,x2,'filled');

grid on;

title('Sequence x_2(n)');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,3);

stem(n,y_add,'filled');

grid on;

title('Addition: x_1(n) + x_2(n)');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,4);

stem(n,y_mul,'filled');

grid on;

title('Multiplication: x_1(n) * x_2(n)');

xlabel('n');

ylabel('Amplitude');

Output Waveform:

Result:

Two discrete-time sequences were generated and their addition and point-wise multiplication

were performed successfully using MATLAB. The resulting sequences were displayed using

stem plots, illustrating basic operations on discrete-time signals.

Experiment 3.2

% Autocorrelation of a Signal

clc;

clear;

close all;

% Input Signal

x = [1 2 3 4 5];

% Autocorrelation

Rxx = xcorr(x);

% Lag values

lags = -(length(x)-1) :(length(x)-1);

% Display result

disp('Autocorrelation Sequence:');

disp(Rxx);

% Plot

figure;

stem(lags,Rxx,'filled');

grid on;

title('Autocorrelation of Signal');

xlabel('Lag');

ylabel('R_{xx}(k)');

Expected Output:

Result:

The autocorrelation of the given discrete-time signal was computed using MATLAB. The

autocorrelation sequence was symmetric about zero lag, and the maximum value occurred at

zero lag, indicating the highest similarity of the signal with itself.