SIMULATION TASK

Experiment

35 Plant G(s) = 1/(0.005s + 0.5) (grid inductor model). PI controller C(s) = Kp + Ki/s. In MATLAB, set Kp=5, Ki=100. Form closed-loop using feedback(series(C,G),1). Plot Bode of open loop and step response of closed loop. Use margin() to print gain and phase margins. Verify steady-state error to step input equals zero (due to integral action).

Program:

clc;

clear;

close all;

%% Plant

% G(s) = 1/(0.005s + 0.5)

G = tf(1, [0.005 0.5]);

%% PI Controller

% C(s) = Kp + Ki/s

Kp = 5;

Ki = 100;

C = pid(Kp, Ki);

%% Open-loop transfer function

L = series(C, G);

%% Closed-loop transfer function

T = feedback(L, 1);

%% Display transfer functions

disp('Plant G(s):');

G

disp('PI Controller C(s):');

C

disp('Open-loop transfer function L(s):');

L

disp('Closed-loop transfer function T(s):');

T

%% Bode plot of open-loop system

figure;

bode(L);

grid on;

title('Bode Plot of Open-Loop System');

%% Gain Margin and Phase Margin

[Gm, Pm, Wcg, Wcp] = margin(L);

fprintf('\n--- Stability Margins ---\n');

if isinf(Gm)

fprintf('Gain Margin = Inf dB\n');

else

fprintf('Gain Margin = %.2f dB\n', 20*log10(Gm));

end

fprintf('Phase Margin = %.2f degrees\n', Pm);

fprintf('Gain Crossover Frequency = %.2f rad/s\n', Wcp);

fprintf('Phase Crossover Frequency = %.2f rad/s\n', Wcg);

%% Step response of closed-loop system

figure;

step(T);

grid on;

title('Closed-Loop Step Response');

xlabel('Time (s)');

ylabel('Output');

%% Steady-state error for unit-step input

ess = abs(1 - dcgain(T));

fprintf('\n--- Steady-State Error ---\n');

fprintf('Steady-state error = %.6f\n', ess);

if ess < 1e-6

fprintf('Steady-state error is approximately ZERO.\n');

fprintf('This is due to the integral action of the PI controller.\n');

else

fprintf('Steady-state error is NOT zero.\n');

end

Output Waveform

Result

The MATLAB simulation of the grid inductor model with PI controller was successfully performed using:

- Plant:

- PI Controller:

The open-loop Bode plot was obtained and the gain margin and phase margin were evaluated using margin(). The closed-loop step response showed that the output reaches the unit reference value.